| COLOR | VALUE | MULTI-PLIER | TOLE- RANCE |
| --- | --- | --- | --- |
| BLACK | 0 | x 1 |  |
| BROWN | 1 | x 10 | ± 1% |
| RED | 2 | x 100 | ± 2% |
| ORANGE | 3 | x 1000 |  |
| YELLOW | 4 | x 10 000 |  |
| GREEN | 5 | x 100 000 |  |
| BLUE | 6 | x 1000 000 |  |
| VIOLET | 7 |  |  |
| GREY | 8 |  |  |
| WHITE | 9 |  |  |
| GOLD |  | x 0.1 | ± 5% |

RESISTOR COLOR CODES

CC BY-SA 4.0     www.picuino.com

- Calculate the missing colors and resistors in the following tables..

- In the last column, calculate the current that will flow through the resistor when connected to a 9 volt battery.

- All values should be written to 3 decimal places and with the multiplier symbol that best corres-ponds to it (M=Mega, k=kilo, m=mili, =micro)

COMMERCIAL SERIES E6:   (5%)

| VALUE  | 1st DIGIT | 2nd DIGIT | MULTI-PLIER | TOLE-RANCE |  | I = 9v / R |
| --- | --- | --- | --- | --- | --- | --- |
| 4.7 M |  |  |  |  |  |  |
|  | RED | RED | BLACK | GOLD |  |  |
| 15  |  |  |  |  |  |  |
|  | ORANGE | ORANGE | GOLD | GOLD |  |  |
| 10 k |  |  |  |  |  |  |
|  | BROWN | GREEN | BROWN | GOLD |  |  |
| 220  |  |  |  |  |  |  |
|  | YELLOW | VIOLET | BLACK | GOLD |  |  |
| 3.3 k |  |  |  |  |  |  |
|  | RED | RED | RED | GOLD |  |  |
| 100  |  |  |  |  |  |  |
|  | BLUE | GREY | BROWN | GOLD |  |  |
| 22 k |  |  |  |  |  |  |
|  | BROWN | BLACK | YELLOW | GOLD |  |  |
| 1.0  |  |  |  |  |  |  |
|  | BLUE | GREY | ORANGE | GOLD |  |  |
| 680 k |  |  |  |  |  |  |
|  | ORANGE | ORANGE | GREEN | GOLD |  |  |

COMMERCIAL SERIES E6:   (5%)

| VALUE  | 1st DIGIT | 2nd DIGIT | MULTI-PLIER | TOLE-RANCE |  | I = 9v / R |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5 k |  |  |  |  |  |  |
|  | YELLOW | VIOLET | RED | GOLD |  |  |
| 2.2  |  |  |  |  |  |  |
|  | BROWN | BLACK | BLACK | GOLD |  |  |
| 2.2 M |  |  |  |  |  |  |
|  | BROWN | GREEN | ORANGE | GOLD |  |  |
| 330 k |  |  |  |  |  |  |
|  | RED | RED | YELLOW | GOLD |  |  |
| 4.7  |  |  |  |  |  |  |
|  | BLUE | GREY | GREEN | GOLD |  |  |
| 470 |  |  |  |  |  |  |
|  | BROWN | BLACK | RED | GOLD |  |  |
| 47 k |  |  |  |  |  |  |
|  | BROWN | GREEN | GOLD | GOLD |  |  |
| 33  |  |  |  |  |  |  |
|  | BROWN | GREEN | GREEN | GOLD |  |  |
| 150 k |  |  |  |  |  |  |
|  | BLUE | GREY | GOLD | GOLD |  |  |
| 68  |  |  |  |  |  |  |
|  | ORANGE | ORANGE | BROWN | GOLD |  |  |
| 1 M |  |  |  |  |  |  |
|  | ORANGE | ORANGE | ORANGE | GOLD |  |  |
| 6.8 k |  |  |  |  |  |  |
|  | YELLOW | VIOLET | YELLOW | GOLD |  |  |

| COLOR | VALUE | MULTI-PLIER | TOLE- RANCE |
| --- | --- | --- | --- |
| BLACK | 0 | x 1 |  |
| BROWN | 1 | x 10 | ± 1% |
| RED | 2 | x 100 | ± 2% |
| ORANGE | 3 | x 1000 |  |
| YELLOW | 4 | x 10 000 |  |
| GREEN | 5 | x 100 000 |  |
| BLUE | 6 | x 1000 000 |  |
| VIOLET | 7 |  |  |
| GREY | 8 |  |  |
| WHITE | 9 |  |  |
| GOLD |  | x 0.1 | ± 5% |

RESISTOR COLOR CODES

CC BY-SA 4.0     www.picuino.com

- Calculate the missing colors and resistors in the following tables..

- In the last column, calculate the current that will flow through the resistor when connected to a 9 volt battery.

- All values should be written to 3 decimal places and with the multiplier symbol that best corres-ponds to it (M=Mega, k=kilo, m=mili, =micro)

COMMERCIAL SERIES E6:   (5%)

| VALUE  | 1st DIGIT | 2nd DIGIT | MULTI-PLIER | TOLE-RANCE |  | I = 9v / R |
| --- | --- | --- | --- | --- | --- | --- |
| 4.7 M | YELLOW | VIOLET | GREEN | GOLD |  | 1,91 A |
| 22  | RED | RED | BLACK | GOLD |  | 409 mA |
| 15  | BROWN | GREEN | BLACK | GOLD |  | 600 mA |
| 3.3  | ORANGE | ORANGE | GOLD | GOLD |  | 2,73 A |
| 10 k | BROWN | BLACK | ORANGE | GOLD |  | 900 A |
| 150  | BROWN | GREEN | BROWN | GOLD |  | 60,0 mA |
| 220  | RED | RED | BROWN | GOLD |  | 40,9 mA |
| 47  | YELLOW | VIOLET | BLACK | GOLD |  | 192 mA |
| 3.3 k | ORANGE | ORANGE | RED | GOLD |  | 2,73 mA |
| 2.2 k | RED | RED | RED | GOLD |  | 4,09 mA |
| 100  | BROWN | BLACK | BROWN | GOLD |  | 90,0 mA |
| 680  | BLUE | GREY | BROWN | GOLD |  | 13,2 mA |
| 22 k | RED | RED | ORANGE | GOLD |  | 409 A |
| 100 k | BROWN | BLACK | YELLOW | GOLD |  | 90,0 A |
| 1.0  | BROWN | BLACK | GOLD | GOLD |  | 9,00 A |
| 68 k | BLUE | GREY | ORANGE | GOLD |  | 132 A |
| 680 k | BLUE | GREY | YELLOW | GOLD |  | 13,2 A |
| 3.3 M | ORANGE | ORANGE | GREEN | GOLD |  | 2,73 A |

COMMERCIAL SERIES E6:   (5%)

| VALUE  | 1st DIGIT | 2nd DIGIT | MULTI-PLIER | TOLE-RANCE |  | I = 9v / R |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5 k | BROWN | GREEN | RED | GOLD |  | 6,00 mA |
| 4.7 k | YELLOW | VIOLET | RED | GOLD |  | 1,91 mA |
| 2.2  | RED | RED | GOLD | GOLD |  | 4,09 A |
| 10  | BROWN | BLACK | BLACK | GOLD |  | 900 mA |
| 2.2 M | RED | RED | GREEN | GOLD |  | 4,09 A |
| 15 k | BROWN | GREEN | ORANGE | GOLD |  | 600 A |
| 330 k | ORANGE | ORANGE | YELLOW | GOLD |  | 27,3 A |
| 220 k | RED | RED | YELLOW | GOLD |  | 40,9 A |
| 4.7  | YELLOW | VIOLET | GOLD | GOLD |  | 1,91 A |
| 6.8 M | BLUE | GREY | GREEN | GOLD |  | 1,32 A |
| 470 | YELLOW | VIOLET | BROWN | GOLD |  | 19,2 mA |
| 1 k | BROWN | BLACK | RED | GOLD |  | 9,00 mA |
| 47 k | YELLOW | VIOLET | ORANGE | GOLD |  | 192 A |
| 1.5  | BROWN | GREEN | GOLD | GOLD |  | 6,00 A |
| 33  | ORANGE | ORANGE | BLACK | GOLD |  | 273 mA |
| 1.5 M | BROWN | GREEN | GREEN | GOLD |  | 6,00 A |
| 150 k | BROWN | GREEN | YELLOW | GOLD |  | 60,0 A |
| 6.8  | BLUE | GREY | GOLD | GOLD |  | 1,32 A |
| 68  | BLUE | GREY | BLACK | GOLD |  | 132 mA |
| 330  | ORANGE | ORANGE | BROWN | GOLD |  | 27,3 mA |
| 1 M | BROWN | BLACK | GREEN | GOLD |  | 9,00 A |
| 33 k | ORANGE | ORANGE | ORANGE | GOLD |  | 273 A |
| 6.8 k | BLUE | GREY | RED | GOLD |  | 1,32 mA |
| 470 k | YELLOW | VIOLET | YELLOW | GOLD |  | 19,2 A |